CAMSHAFTS  AND LIFTERS

In choosing a CAMSHAFT for your 4X4 vehicle and tow vehicles, your ultimate goal is to increase both horsepower and torque within a useable RPM range to allow the engine to operate with the least amount of vacuum throttle opening. The result - better fuel economy, smoother idling, cooler running, and longer engine life.

Jeep 2.5-4.2-4.0 Engines

| Grind Part* | RPM Range | Advertised Duration | Duration @.050 | Lobe Centers Available |  | 1.6 Int/Exh |  | Comments: |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HYDRAULIC |  |  |  |  |  |  |  |  |
|  | 1500 |  |  |  |  |  |  |  |
| RV-O-B |  | 264 | 214 | 110" |  | 470 |  | G Good torque & mileage - smooth Idle |
| H-264 | to | 264 | 214 | 108°-112° |  | 470 |  | Light towing - good performance & mileage - smooth Idle |
| H-6474 |  | 264/274 | 214/224 | 108M12" |  | 470/480 |  | Towing - good performance -restricted exhaust (low smooth idle |
| H-268 | 5200 | 268 | 21.6 | 108°-112° |  | 480 |  | Towing - wide performance range -smooth idle |
| H-274 |  | 274 | 224 | 108°-112° |  | 480 |  | Hi-performance street - fair vacuum - healthy exhaust sound |
| H-7464 |  | 274/264 | 224/214 | 108°-112° |  | 480/470 |  | Towing - Turbo charged - good performance - smooth idle |
| SOLID |  |  |  |  |  |  |  |  |
| C-265 | 1500 5200 | 265 | 224 | 106°-112° |  | 460 |  | Good performance & mileage |
|  | to |  |  |  |  |  |  |  |
| C-272 | 5200 | 272 | 224 | 106°-112° |  | 464 |  | Good all around power, higher horsepower curve. |

Also available for Chevrolet, Ford and Dodge V-8 Engine except hydraulic roller

The use of incorrect springs with these cams could result in severe engine damage. Use only recommended valve springs.